Section 4: Cycle-Counted Phase Delay, Task 4 (2 pages plus figures)

Include:

• The Assembly listing, commented per instruction with the cycle cost of each instruction.

/*

* dsp.s

* EEE3096S 2026 - Practical 1B, Task 4

* Cycle-counted ADC to DAC loop with a 45 degree phase delay

*

* Student 1 : Joshua Handyside HNDJOS012

* Student 2 : Tebogo Teffo TFFTEB003

*/

.syntax unified

.thumb

.cpu cortex-m0

.fpu softvfp

.global DSP_Loop

.type DSP_Loop, %function

@ ---------------------------------------------------------------------------

@ Peripheral addresses

@ ---------------------------------------------------------------------------

.equ ADC_DR, 0x40012440

.equ DAC_DHR12R1, 0x40007408

.section .text.DSP_Loop, "ax", %progbits

@ ===========================================================================

@ ENTRY POINT

@ ===========================================================================

DSP_Loop:

@ Setup registers outside the timed loop

LDR R0, =ADC_DR

LDR R1, =DAC_DHR12R1

loop:

@ --- SAMPLE AND OUTPUT ------------------------------------------------

@ read the current ADC conversion from the Data Register

LDR R2, [R0] @ 2 cycles

@ write the value straight out to the DAC Data Register

STR R2, [R1] @ 2 cycles

@ --- DELAY SETUP ------------------------------------------------------

@ for a 45 degree phase shift, we need a 125us delay,

@ or 1000 cycles

@ LDR + STR + MOVS = 5 cycles

@ last lööp = 2 cycles

@ lööp branch = 3 cycles

@ this leaves us with 990 cycles

@ we add 2 NOPs for padding,

@ then run the lööp 248 times (988/4)+1=248

MOVS R3, #248 @ 1 cycle - loading lööp counter for 248 lööps

NOP @ 1 cycle - hold up

NOP @ 1 cycle - wait a minute

@ fill my cup, put some liquor in it

delay_loop:

@ --- INNER LOOP -------------------------------------------------------

@ implement the counted delay loop

SUBS R3, R3, #1 @ 1 cycle - decrement counter with flag update

BNE delay_loop @ 3 cycles; 1 cycle on exit

@ --- REPEAT -----------------------------------------------------------

@ branch back to the start of the main 'loop'.

B loop @ 3 cycles - always jump back in the lööp

@ ----------------------------------------------------------------------

@ NOTE: You must calculate your exact cycle budget, showing the cost

@ of every instruction and loop iteration, and document it in your report.

@ ----------------------------------------------------------------------

.size DSP_Loop, .-DSP_Loop

• A cycle budget table containing the instruction, cycles per instruction, iteration count, subtotal, and running total.

| Instruction | Cycles per Instruction | Iteration Count | Subtotal | Running Total |
| --- | --- | --- | --- | --- |
| LDR | 2 | 1 | 2 | 2 |
| STR | 2 | 1 | 2 | 4 |
| MOV | 1 | 1 | 1 | 5 |
| NOP | 1 | 2 | 2 | 7 |
| SUBS | 1 | 248 | 248 | 255 |
| BNE | 3 | 247 | 741 | 996 |
| BNE (exit) | 1 | 1 | 1 | 997 |
| B | 3 | 1 | 3 | 1000 |

• The derivation of 125 μs from the 8 MHz system clock and the 1 kHz input.

T = 1/f =  1/1000 = 1ms

∆t = (45°/360°)T = (1/8)(1ms) = 125μs

• The oscilloscope screenshot with CH1 connected to PB0, CH2 connected to PA4, and cursors placed on the measured time difference.

• The measured ∆t, the error relative to 125 μs, and the source of the error.

∆t = 126μs. The error of 1μs is a result of the cursor having a resolution of 2μs. Visually, the value seems to fall between the measurable points at 124μs and 126μs.

Section 5: Analog Debugging of the LCD Enable Line, Task 5 (3 pages plus figures)

Include:

• The Assembly subroutine for 4-bit initialisation and displaying the letter “A”.

@ ===========================================================================

@ ENTRY POINT

@ ===========================================================================

LCD_Run:

PUSH {LR}

@ wait for the LCD power rail to settle (usually 40ms, wait for 50ms)

MOVS R0, #50

BL LCD_DelayLong

@ call the 4-bit initialization sequence

BL LCD_Init

@ write the character 'A' (0x41) to the display.

MOVS R0, #0x41

BL LCD_WriteData

hang:

B hang

.size LCD_Run, .-LCD_Run

@ ===========================================================================

@ LCD_Init

@ Puts the controller into 4-bit mode and readies the display.

@ ===========================================================================

.type LCD_Init, %function

LCD_Init:

PUSH {LR}

@ send the 4-bit initialization sequence

@ referenced from the HD44780 datasheet flowchart

@ send commands with RS low using LCD_WriteCmd

LDR R0, =GPIOC_BSRR

LDR R1, =0x40000000 @ reset RS

STR R1, [R0]

MOVS R0, #0x03

BL LCD_SendNibble @ first wakeup command

BL LCD_Pulse

MOVS R0, #5

BL LCD_DelayLong @ wait for more than 4.1ms

MOVS R0, #0x03

BL LCD_SendNibble @ second wakeup command

BL LCD_Pulse

MOVS R0, #150

BL LCD_DelayShort @ wait for more than 100us

MOVS R0, #0x03

BL LCD_SendNibble @ third wakeup command

BL LCD_Pulse

MOVS R0, #100

BL LCD_DelayShort

MOVS R0, #0x02

BL LCD_SendNibble @ switching to 4 bit mode

BL LCD_Pulse

MOVS R0, #100

BL LCD_DelayShort

MOVS R0, #0x28

BL LCD_WriteCmd

MOVS R0, #0x08 @ turn off display

BL LCD_WriteCmd

MOVS R0, #0x01 @ clear display

BL LCD_WriteCmd

MOVS R0, #2 @ wait more than 1.52ms for clear

BL LCD_DelayLong

MOVS R0, #0x06 @ set entry mode to increment cursor

BL LCD_WriteCmd

MOVS R0, #0x0C @ turn on display and turn off cursor

BL LCD_WriteCmd

POP {PC}

• The failing case, with a screenshot of PC15 and PC15_S together showing the 5 V side falling short of 3.5 V.

Even without an implemented time delay fix, there is no noticeable delay between PC15 and PC15_S.

• The measured rise time from 0 V to 3.5 V on PC15 S, with cursors.

On PC15_S, the measured rise time as seen in the above figure is 80ns.

• The capacitance calculation, stating R, VCC, and VC (t), using: C = −t/(R ln(1-(Vc/Vcc))).

R = 1800Ω

Vcc = 5V

Vc(t) = 3.5V

t=80ns

C = 36.91pF

• The NOP count derivation, showing the measured rise time and the 450 ns HD44780 hold requirement converted to cycles at 125 ns per cycle.

Rise time = 80ns = 1 cycle

Hold requirement = 450/125 = 4 cycles

NOP count = 5

• The fixed case, with a screenshot showing PC15 S above 3.5 V for at least 450 ns.

• A photograph of the LCD displaying “A”.